PROJECT FACT SHEET — DOCUMENT 00.2  |  DATA ROOM

Jingo Gold Project

Quick-Reference Project Parameters  —  Elmore County, Idaho

PROPERTY

| Project Name | Jingo Gold Project |
| --- | --- |
| Jurisdiction | Idaho, U.S.A. |
| County | Elmore County |
| Mining District | Pine Grove Mining District |
| Closest Town | Fairfield, Idaho (~21 miles E) |
| Distance to Major City | ~54 road miles N-NE of Mountain Home; ~50 km from Boise |
| Project Centroid | 43.4060°N, 115.1966°W (NAD 83) |
| Access | Road directly to property; year-round access |
| Elevation Range | Approximately 5,000–6,000 ft |

LAND PACKAGE

| Total Area | ~261 acres (~106 hectares) |
| --- | --- |
| Claim Type | Unpatented mining claims (federal BLM land) |
| Claim Groups | Jingo Group, Hornett, Velvet, Good Life |
| Coverage Intent | Full known vein strike + along-strike extensions |
| Tenure Status | Active — federal annual maintenance required |

COMMODITIES & GRADES (EXPLORATION STAGE)

| Source | Au (oz/t) | Ag (oz/t) | Type / Notes |
| --- | --- | --- | --- |
| 1933 Production (153 t) | 1.28 | 6.70 | Bulk shipment — historic shipping grade |
| Sample 6803 (Oct 1985) | 1.057 | 11.25 | Grab — bug hole quartz, peak modern |
| Sample 6811 (Oct 1985) | 0.734 | 8.18 | Grab — white-gray quartz, Fe spider web |
| Sample 878 (May 1986) | 0.802 | 1.95 | Grab — high-grade FeOx-stained dump rock |
| Sample 877 (May 1986) | 0.532 | 0.27 | Channel cut — 8' of qtz vein with HW gouge |
| Sample 6810 (Oct 1985) | 0.299 | 2.86 | Grab — fine grain quartz, Cu stains |
| 1985 Avg (7 samples) | 0.109 | 0.65 | Average of August 1985 program |

GEOLOGY

| Deposit Style | Epithermal bull-quartz fissure vein system |
| --- | --- |
| Host Rocks | Granodiorite / quartz monzodiorite (Idaho Batholith) |
| Adjacent Lithologies | Cretaceous granite (north); younger dacites (south); olivine tholeiite basalt regionally |
| Vein Strike | ~N60°E |
| Vein Dip | 75°–90° South (steep) |
| Vein Width | 1–6 ft typical; up to 10 ft near workings |
| Vein Length (mapped) | Long, discontinuous along strike — ~1 km mapped |
| Sulphide Minerals | Pyrite (disseminated), galena, sphalerite, occasional Cu sulphide |
| Key Texture | "Bug hole" quartz with spider-web iron staining hosts highest grades |
| Alteration | Silicification of granite hanging wall (also carries gold) |

EXPLORATION HISTORY

| 1893 | Original discovery (Jumbo and Hercules claims) |
| --- | --- |
| 1903 | Salt Lake City company operated under bond; 600 ft of development on the fissure vein |
| 1933 | Shipment of 153 tons → 1.28 oz/t Au, 6.70 oz/t Ag |
| Aug 1985 | 7-sample channel and dump sampling program (Boise Assayer) |
| Oct 1985 | 13-sample detailed grab program — peak result 1.057 oz/t Au + 11.25 oz/t Ag |
| May 1986 | 12-sample program — 8' channel cut at 0.532 oz/t Au; high-grade grabs to 0.802 oz/t Au |
| 2025–26 | Property assembled and presented for transaction |

INFRASTRUCTURE

| Road Access | Direct road access to property |
| --- | --- |
| Power | Idaho grid accessible — area is populated rural |
| Water | Local watershed (South Fork Boise River basin) |
| Nearest Town | Fairfield (~21 miles E) |
| Nearest Airport (commercial) | Boise (~50 km SW) |
| Nearest Mill / Processing | Multiple historic and active operations in district |

ADJACENT / REGIONAL CONTEXT

| Project / Operator | Stage | Notes |
| --- | --- | --- |
| Stibnite Gold (Perpetua Resources) | Permitted dev. | World-class Au-Sb project; +4 Moz Au reserve |
| Beartrack-Arnett (Revival Gold) | Past producer / dev. | Multi-million oz Au resource; PFS stage |
| DeLamar (Integra Resources) | PFS/FS stage | Past Au-Ag producer; PFS-stage resource |
| Black Pine (Liberty Gold) | PFS stage | Au heap-leach restart project |
| Last Chance (historic) | Historic mine | Au-Ag, same regional structural trend NE of Jingo |
| Lucky Friday (Hecla) | Active mine | World-class Ag producer (Coeur d'Alene district) |

Contact: Hugo  |  hugo@kpcapital.com  |  778-985-8685

All historic assays pre-date modern NI 43-101 / SK 1300 QA/QC. Grades are exploration indicators only. Adjacent project data is from public operator disclosures.